1.List out two differences between stimulated and spontaneous emission.

Spontaneous emission does not require an external electromagnetic stimulus to release energy, whereas stimulated emission does require external electromagnetic stimuli to release energy. During spontaneous emission, only one energy wave is released, but during stimulated emission two energy waves are released.

2. What is mean by meta stable state?

Metastable state is an excited state of an atom or other system with a longer lifetime than the other excited states. ... A large number of excited atoms are accumulated in the metastable state.

3. List ant four advantages of using semiconductor laser?

The advantages of semiconductor lasers are: small volume, lightweight, good reliability, long service life, and low power consumption.

4. State the importance of modulation of Laser source

Modulation allows us to send a signal over a bandpass frequency range. If every signal gets its own frequency range, then we can transmit multiple signals simultaneously over a single channel, all using different frequency ranges. Another reason to modulate a signal is to allow the use of a smaller antenna.

5. List out the various noise sources in optical receiver circuit.

The main noises associated with p-i-n photodiode receivers are quantum or shot noise and dark current associated with photodetection, and thermal noise occurring in the electronics

6. Define modulation of Optical sources

An optical modulator is a device which is used to modulate a beam of light. ... Often the easiest way to obtain modulation of intensity of a light beam, is to modulate the current driving the light source, e.g. a laser diode.

7. State the importance of front end amplifier in optical receivers.

The need for high speed optical receivers are being driven by high speed and wide bandwidth optical communications demands. The front-end amplifier is a critical part in optical receiver. ... Its transimpedance gain is 4 kQ The LA bandwidth is 586.8MHz and the output swing is 4.2V.